pfSense VLAN Segmentation Lab

| Role | Network Administrator / Security Engineer |
| --- | --- |
| Scope | - Implement VLANs (Virtual LANs) for HR and IT using a single pfSense interface. - Configure IP addressing and DHCP for both VLANs. - Apply firewall rules to control inter-VLAN traffic. - Demonstrate and verify network segmentation using client VMs. |
| Prerequisits | - VirtualBox environment with: - pfSense Firewall VM (Main pfSense instance) - Two client VMs: - HR-Client (existing client renamed) - IT-Client (clone of HR-Client) - Network Configuration: - pfSense LAN Adapter 3 → hr-net - pfSense LAN Adapter 4 → it-net - HR-Client → connected to hr-net - IT-Client → connected to it-net - Basic understanding of IP addressing, DHCP, and firewall configuration. |

Objective

To create two logically separate VLANs for HR and IT departments using a single network interface on pfSense and configure firewall rules to control inter-network traffic, demonstrating effective network segmentation.

Tasks

### 1.  Creating VLANs in pfSense (HR & IT)

Goal: Define the VLAN tags that pfSense will use to separate traffic.

- Log into your pfSense web interface as the admin user.

- Navigate to Interfaces > Assignments.

- Click on the VLANs tab.

- Click the + Add button to create your first VLAN.

  - Parent Interface: Select your LAN interface (e.g., em1).

  - VLAN Tag: 10 (unique ID for HR network).

  - Description: HR Network.

  - Click Save.

- Click + Add again to create the second VLAN.

  - Parent Interface: Select the same LAN interface (e.g., em1).

  - VLAN Tag: 20 (unique ID for IT network).

  - Description: IT Network.

  - Click Save.

- You have now defined the VLANs, but they don't do anything until they are assigned and configured.

### 2 . Interface Assignments and Configuration

Goal: Turn the virtual VLANs into usable pfSense interfaces with IP addresses.

- Navigate back to Interfaces > Assignments.

- Under Available network ports, you will now see your two new VLANs.

- Click the + Add button next to VLAN 10 on em1 to create a new interface called OPT1.

- Click the + Add button next to VLAN 20 on em1 to create another new interface called OPT2.

Configure OPT1 (HR Department):

- Click on OPT1.

- Check the box for Enable interface.

- Change the Description to HR.

- Set IPv4 Configuration Type to Static IPv4.

- Under Static IPv4 Configuration, set the IPv4 Address to 192.168.10.1/24.

- Click Save, then Apply Changes.

Configure OPT2 (IT Department):

- Go back to Interfaces > Assignments and click on OPT2.

- Check the box for Enable interface.

- Change the Description to IT.

- Set IPv4 Configuration Type to Static IPv4.

- Set the IPv4 Address to 192.168.20.1/24.

- Click Save, then Apply Changes.

### 3 . DHCP Setup per VLAN

Goal: Configure pfSense to automatically assign IP addresses to devices on both the HR and IT networks.

- Navigate to Services > DHCP Server.

- You will now see tabs for your new interfaces: HR and IT.

Configure DHCP for HR VLAN:

- Click on the HR tab.

- Check the box for Enable DHCP server on HR interface.

- Set IP Range: 192.168.10.100 to 192.168.10.200.

- Click Save.

Configure DHCP for IT VLAN:

- Click on the IT tab.

- Check the box for Enable DHCP server on IT interface.

- Set IP Range: 192.168.20.100 to 192.168.20.200.

- Click Save.

### 4 .  Firewall Rule Configuration for Inter-VLAN Restrictions

Goal: Implement the core security policy — IT can access the HR network, but HR cannot access the IT network.

- Navigate to Firewall > Rules.

Allow Internet Access:

- Click on the HR tab → + Add.

  - Create a rule that allows traffic from HR net to any destination.

  - Click Save.

- Click on the IT tab → + Add.

  - Create a similar rule allowing traffic from IT net to any destination.

  - Click Save.

Allow IT to Access HR Network:

- While on the IT tab, click + Add.

  - Action: Pass

  - Interface: IT

  - Protocol: any

  - Source: IT net

  - Destination: Single host or alias → HR net

  - Description: Allow IT to access HR network.

  - Click Save, then Apply Changes.

Note: No block rule is needed on the HR interface.
 By default, pfSense blocks all inter-interface traffic unless a “Pass” rule is defined.

### 5 .  Testing Segmentation with Ping and Access Attempts

Goal: Verify that your firewall rules are working exactly as intended.

- Start both HR-Client and IT-Client VMs.

- Check IP Addresses:

  - On HR-Client, run ip a — should show IP in 192.168.10.x.

  - On IT-Client, run ip a — should show IP in 192.168.20.x.

- Perform connectivity tests:

  - From HR-Client → ping 8.8.8.8 → Should succeed.

  - From IT-Client → ping 8.8.8.8 → Should succeed.

- Inter-VLAN tests:

  - From IT-Client → ping 192.168.10.100 (HR client) → Should succeed.

  - From HR-Client → ping 192.168.20.100 → Should fail.

Outcome

- Successfully created VLANs (HR: VLAN 10, IT: VLAN 20) on pfSense.

- Each VLAN has its own subnet and DHCP service.

- Both VLANs have internet access.

- IT can access HR, but HR cannot access IT.

- Validated segmentation through ping and access tests.

- Demonstrated secure network isolation and VLAN-based segmentation.

Solution

1.  Creating VLANs in pfSense (HR & IT)

Goal: Define the VLAN tags that pfSense will use to separate traffic.

- Log into your pfSense web interface as the admin user.

- Navigate to Interfaces > Assignments.

- Click on the VLANs tab.

- Click the + Add button to create your first VLAN.

  - Parent Interface: Select your LAN interface (e.g., em1). This is the physical port the tagged traffic will travel on.

  - VLAN Tag: 10. This is the unique ID for the HR network.

  - Description: HR Network.

  - Click Save.

- Click + Add again to create the second VLAN.

  - Parent Interface: Select the same LAN interface (e.g., em1).

  - VLAN Tag: 20. The unique ID for the IT network.

  - Description: IT Network.

  - Click Save.

You have now defined the VLANs, but they don't do anything until they are assigned and configured.

2. Interface Assignments and Configuration

Goal: Turn the virtual VLANs into usable pfSense interfaces with IP addresses.

- Navigate back to Interfaces > Assignments.

- Under Available network ports, you will now see your two new VLANs.

- Click the + Add button next to VLAN 10 on em1 to create a new interface called OPT1.

- Click the + Add button next to VLAN 20 on em1 to create another new interface called OPT2.

- Now, let's configure OPT1 for the HR department. Click on the OPT1 name.

  - Check the box for Enable interface.

  - Change the Description to HR.

  - Set IPv4 Configuration Type to Static IPv4.

  - Under Static IPv4 Configuration, set the IPv4 Address to 192.168.10.1 and the subnet mask to /24.

  - Click Save, and then Apply Changes.

- Next, configure OPT2 for the IT department. Go back to Interfaces > Assignments and click on OPT2.

  - Check the box for Enable interface.

  - Change the Description to IT.

  - Set IPv4 Configuration Type to Static IPv4.

  - Set the IPv4 Address to 192.168.20.1 with a /24 subnet mask.

  - Click Save, and then Apply Changes.

3.  DHCP Setup per VLAN

Goal: Configure pfSense to automatically assign IP addresses to devices on both the HR and IT networks.

- Navigate to Services > DHCP Server.

- You will now see tabs for your new interfaces: HR and IT.

- Click on the HR tab.

  - Check the box for Enable DHCP server on HR interface.

  - Set the Range of IP addresses to issue, for example, from 192.168.10.100 to 192.168.10.200.

  - Click Save.

- Now click on the IT tab.

  - Check the box for Enable DHCP server on IT interface.

  - Set the Range from 192.168.20.100 to 192.168.20.200.

  - Click Save.

4.  Firewall Rule Configuration for Inter-VLAN Restrictions

Goal: Implement the core security policy: IT can access the HR network, but HR cannot access the IT network.

- First, we need to give both networks internet access.

  - Navigate to Firewall > Rules.

  - Click the HR tab. Click + Add.

  - Create a rule that allows traffic from HR net to any destination. This is a basic "allow out" rule. Click Save.

  - Click the IT tab. Click + Add.

  - Create a similar rule allowing traffic from IT net to any destination. Click Save.

- Now, create the specific inter-VLAN rule.

  - While on the IT tab, click + Add to create a new rule.

  - Action: Pass

  - Interface: IT

  - Protocol: any

  - Source: IT net

  - Destination: Select Single host or alias and type HR net.

  - Description: Allow IT to access HR network.

  - Click Save, then Apply Changes.

Important: We do NOT need to create a block rule on the HR interface. By default, pfSense blocks all traffic between interfaces unless a "Pass" rule specifically allows it. Since we never created a rule allowing HR to talk to IT, traffic is already blocked.

5.  Testing Segmentation with Ping and Access Attempts

Goal: Verify that your firewall rules are working exactly as intended.

- Start both client VMs: Power on HR-Client and IT-Client.

- Check IP Addresses:

  - On HR-Client, open a terminal and type ip a. It should have an IP address in the 192.168.10.x range.

  - On IT-Client, do the same. It should have an IP in the 192.168.20.x range.

- Perform the Tests from HR-Client:

  - In the terminal, test internet access: ping 8.8.8.8. This should succeed.

- Perform the Tests from IT-Client:

  - In the terminal, test internet access: ping 8.8.8.8. This should succeed.

  - Now, attempt to access the HR network: ping 192.168.10.100 (the HR-Client's IP). This should be SUCCEED.